Session 7

-

ADC and UART serial communication

Author: Guillermo Cortés Orellana

Teacher: Tomáš Frýza

Lab assignment

- Preparation tasks

- Table with voltaje divider, calculated and measured ADC values for all buttons

| Push button | PC0[A0] voltage | ADC value (calculated) | ADC value (measured) |
| --- | --- | --- | --- |
| Right | 0 V | 0 | 0 |
| Up | 0.495 V | 101 | 101 |
| Down | 1.2 V | 245 | 245 |
| Left | 1.96 V | 401 | 402 |
| Select | 3.18 V | 650 | 650 |
| None | 5 V | 1023 | 1022 |

- ADC

- Listing of ADC_vect interrupt routine with complete code for sending data to the LCD/UART and identification of the pressed Button

You can find the code on my GitHub:

https://github.com/GuicoRM/Digital-Electronics-2

- Screenshot of SimulIDE circuit when “Power Circuit” is applied

- UART

- (Hand-drawn) picture of UART signal when transmitting data DE2 in 4800 7O2 mode (7 data bits, odd parity, 2 stop bits, 4800 Bd)

If we consider 4800 Bd  → TBIT = 14800 → TBIT = 208 us

DE2 = 100 0100 – 100 0101 – 011 0010 (with 7 bits)

- Listing of code for calculating/displaying parity bit

I couldn’t find out the solution of the code. I have several ideas about how to solve it but I don’t know how to codify them.

- We could work with frame data

- We could work with the variable ‘value’ which is equals to ‘ADC’ in order to count the number of ‘1’ and ‘0’ and find out the parity

- We could solve the problem working  in rudimentary way which only would be valid for this example: first, we could display different values of voltage; secondly we could count the number of ‘1’ and ‘0’ and finally decide if we chose odd or even parity and display it